Circuit Documentation

Summary

This circuit is a complex system involving a Seeeduino XIAO microcontroller, two ULN2803 Darlington Arrays, multiple coin vibration motors, a BNO085 sensor, an MCP23S17 I/O expander, and a push button. The circuit is designed to control multiple vibration motors through the Darlington arrays, which are driven by the I/O expander. The Seeeduino XIAO serves as the main controller, interfacing with the I/O expander and the BNO085 sensor for additional functionality. The push button is used for user input.

Component List

- Seeeduino XIAO

- Description: A compact microcontroller board based on the SAMD21G18 chip.

- Pins: A0 - D0 - DAC, A1 - D1, A2 - D2, A3 - D3, A4 - D4 - SDA, A5 - D5 - SCL, A6 - D6 - TX, 5V, GND, 3V3, A10 - D10 - MOSI, A9 - D9 - MISO, A8 - D8 - SCK, A7 - D7 - RX

- ULN2803 Darlington Array

- Description: A high-voltage, high-current Darlington transistor array.

- Pins: I1, I2, I3, I4, I5, I6, I7, I8, O1, O2, O3, O4, O5, O6, O7, O8, GND, COMMON

- COIN VIBRATION MOTOR

- Description: A small, compact motor used for haptic feedback.

- Pins: +, -

- BNO085

- Description: A 9-axis sensor providing orientation and motion data.

- Pins: VCC, GND, SCL/SCK/RX, SDA/MISO/TX, ADR/MOSI, CS, INT, RST, PS1, PS0

- MCP23S17

- Description: A 16-bit I/O expander with SPI interface.

- Pins: GPB0, GPB1, GPB2, GPB3, GPB4, GPB5, GPB6, GPB7, A1, A0, SO, SI, CS, VSS, VDD, INTA, INTB, RESET, A2, GPA1, GPA0, GPA5, GPA4, GPA3, GPA2, GPA7, GPA6, SCK

- Push Button

- Description: A simple push button for user input.

- Pins: pin1, pin2

Wiring Details

Seeeduino XIAO

- A7 - D7 - RX is connected to CS of MCP23S17.

- A8 - D8 - SCK is connected to SCK of MCP23S17.

- A10 - D10 - MOSI is connected to SI of MCP23S17.

- A9 - D9 - MISO is connected to SO of MCP23S17.

- A5 - D5 - SCL is connected to SCL/SCK/RX of BNO085.

- A4 - D4 - SDA is connected to SDA/MISO/TX of BNO085.

- A6 - D6 - TX is connected to pin2 of the push button.

- 3V3 is connected to VCC of BNO085 and VDD of MCP23S17.

- GND is connected to GND of BNO085, pin1 of the push button, GND of both ULN2803 Darlington Arrays, and VSS of MCP23S17.

ULN2803 Darlington Array (1)

- I1 is connected to GPA7 of MCP23S17.

- I2 is connected to GPA6 of MCP23S17.

- I3 is connected to GPA5 of MCP23S17.

- I4 is connected to GPA4 of MCP23S17.

- I5 is connected to GPA3 of MCP23S17.

- I6 is connected to GPA2 of MCP23S17.

- I7 is connected to GPA1 of MCP23S17.

- I8 is connected to GPA0 of MCP23S17.

- COMMON is connected to the positive terminals of multiple coin vibration motors.

ULN2803 Darlington Array (2)

- I1 is connected to GPB7 of MCP23S17.

- I2 is connected to GPB6 of MCP23S17.

- I3 is connected to GPB5 of MCP23S17.

- I4 is connected to GPB4 of MCP23S17.

- I5 is connected to GPB3 of MCP23S17.

- I6 is connected to GPB2 of MCP23S17.

- I7 is connected to GPB1 of MCP23S17.

- I8 is connected to GPB0 of MCP23S17.

- COMMON is connected to the positive terminals of multiple coin vibration motors.

COIN VIBRATION MOTORS

- The negative terminals of the motors are connected to the outputs of the ULN2803 Darlington Arrays.

BNO085

- GND is connected to the common ground.

- VCC is connected to the 3.3V supply.

- SCL/SCK/RX and SDA/MISO/TX are connected to the corresponding pins on the Seeeduino XIAO.

MCP23S17

- CS, SCK, SI, and SO are connected to the corresponding pins on the Seeeduino XIAO.

- VDD is connected to the 3.3V supply.

- VSS is connected to the common ground.

Push Button

- pin1 is connected to the common ground.

- pin2 is connected to A6 - D6 - TX of the Seeeduino XIAO.