// Content from Button.cpp

#include "Button.h"

#include <Arduino.h>

Button* Button::instance = nullptr;

Button::Button(int pin, CompassBelt* compassBelt) : button(pin, true, false), compassBelt(compassBelt) {

instance = this;

}

void Button::setup() {

button.attachDoubleClick(doubleclick);

button.attachClick(singleclick);

button.attachLongPressStop(longclick);

button.setDebounceMs(20);

button.setClickMs(400);

button.setPressMs(2000);

}

void Button::tick() {

button.tick();

}

void Button::doubleclick() {

instance->compassBelt->setAlwaysOn(!instance->compassBelt->isAlwaysOn());

Serial.println("Double click");

}

void Button::singleclick() {

instance->compassBelt->setNextVibrationInterval();

Serial.println("Single click");

}

void Button::longclick() {

instance->compassBelt->lampTest();

Serial.println("long click");

}

// Content from Button.h

#ifndef BUTTON_H

#define BUTTON_H

#include "OneButton.h"

#include "CompassBelt.h"

class Button {

public:

Button(int pin, CompassBelt* compassBelt);

void setup();

void tick();

private:

OneButton button;

CompassBelt* compassBelt;

static Button* instance;

static void doubleclick();

static void singleclick();

static void longclick();

};

#endif // BUTTON_H

// Content from Compass.cpp

#include "Compass.h"

void Compass::quaternionToEuler(float qr, float qi, float qj, float qk, euler_t* ypr, bool degrees) {

float sqr = sq(qr);

float sqi = sq(qi);

float sqj = sq(qj);

float sqk = sq(qk);

ypr->yaw = atan2(2.0 * (qi * qj + qk * qr), (sqi - sqj - sqk + sqr));

ypr->pitch = asin(-2.0 * (qi * qk - qj * qr) / (sqi + sqj + sqk + sqr));

ypr->roll = atan2(2.0 * (qj * qk + qi * qr), (-sqi - sqj + sqk + sqr));

if (degrees) {

ypr->yaw *= RAD_TO_DEG;

ypr->pitch *= RAD_TO_DEG;

ypr->roll *= RAD_TO_DEG;

}

}

/*

void Compass::quaternionToEulerRV(sh2_RotationVector_t* rv, euler_t* ypr, bool degrees)

{

// Read quaternion (adjust field names if your struct differs)

const float qi = rv->i;

const float qj = rv->j;

const float qk = rv->k;

const float qw = rv->real;   // sometimes rv->w

// ... then do the same quaternion->euler math you already do

// (copy/paste from your WAcc version, just replacing the input fields)

}

*/

float Compass::getNorthDirection(float yaw) {

if (yaw < 0) {

return (360 + yaw);

} else {

return yaw;

}

}

float Compass::ConvertToShownDirection(float AbsDir, int degree_shift) {

return ((int(AbsDir) + degree_shift) % 360);

}

void Compass::quaternionToEulerRV(sh2_RotationVectorWAcc_t* rotational_vector,

euler_t* ypr,

bool degrees) {

quaternionToEuler(rotational_vector->real, rotational_vector->i, rotational_vector->j, rotational_vector->k, ypr, degrees);

}

void Compass::quaternionToEulerGI(sh2_GyroIntegratedRV_t* rotational_vector,

euler_t* ypr,

bool degrees) {

quaternionToEuler(rotational_vector->real, rotational_vector->i, rotational_vector->j, rotational_vector->k, ypr, degrees);

}

void Compass::updateFromSensor(const sh2_SensorValue_t& value) {

sensorValue_1 = value;

update_sensor_1 = true;

}

float Compass::getHeading(BNO08x* imu, int degree_shift) {

if (update_sensor_1) {

if (reportType == SH2_ROTATION_VECTOR) {

quaternionToEulerRV(&sensorValue_1.un.rotationVector, &ypr_1, true);

}

//        } else if (reportType == SH2_GAME_ROTATION_VECTOR) {

//            quaternionToEulerRV(&sensorValue_1.un.gameRotationVector, &ypr_1, true);

//        }

update_sensor_1 = false;

}

float absDir = getNorthDirection(ypr_1.yaw);

float shownDir = ConvertToShownDirection(absDir, degree_shift);

return shownDir;

}

void Compass::setReports(BNO08x* imu, sh2_SensorId_t reportType, long report_interval)

{

if (!imu->enableReport(reportType, report_interval))

{

Serial.println("Could not enable rotation vector on BNO08x");

}

}

// Content from Compass.h

#ifndef COMPASS_H

#define COMPASS_H

#include <Arduino.h>

#include <Wire.h>

#include <sh2_SensorValue.h>

#include <SparkFun_BNO08x_Arduino_Library.h>

struct euler_t {

float yaw;

float pitch;

float roll;

};

class Compass {

public:

Compass() : update_sensor_1(false),   reportType(SH2_GAME_ROTATION_VECTOR), reportIntervalUs(100000L) {}

float getHeading(BNO08x* bno08x, int degree_shift);

void updateFromSensor(const sh2_SensorValue_t& value);

uint8_t getAccuracy() const { return sensorValue_1.status; }

long getReportInterval() const { return reportIntervalUs; }

static void setReports(BNO08x* bno08x, sh2_SensorId_t reportType, long report_interval);

private:

euler_t ypr_1;

sh2_SensorValue_t sensorValue_1;

bool update_sensor_1;

sh2_SensorId_t reportType;

long reportIntervalUs;

static void quaternionToEuler(float qr, float qi, float qj, float qk, euler_t* ypr, bool degrees);

static void quaternionToEulerRV(sh2_RotationVectorWAcc_t* rotational_vector, euler_t* ypr, bool degrees);

static void quaternionToEulerGI(sh2_GyroIntegratedRV_t* rotational_vector, euler_t* ypr, bool degrees);

//    static void quaternionToEulerRV(sh2_RotationVector_t* rotational_vector,

//                                euler_t* ypr, bool degrees);

float getNorthDirection(float yaw);

float ConvertToShownDirection(float AbsDir, int degree_shift);

};

#endif // COMPASS_H

// Content from CompassBelt.cpp

#include "CompassBelt.h"

const long CompassBelt::vibrationIntervalChoices[] = {500, 1500, 2500, 3500};// The default

CompassBelt::CompassBelt(unsigned long vibrationDuration,

unsigned long vibrationInterval,

uint8_t chipSelect)

: _vibrationDuration(vibrationDuration),

_vibrationInterval(vibrationInterval),

_chipSelect(chipSelect),

_bank(&SPI, chipSelect, 0),

_haptic(_bank)

{

vibrationInterval_ = vibrationInterval;

lastDirection_ = -1;

onDuration_ = vibrationDuration;

vibrationInterval_ = vibrationInterval;

lastOn_ = 0;

}

void CompassBelt::setupPins() {

_bank.begin();

_haptic.begin();

for (int i = 0; i < 16; i++) {

_bank.pinMode(i,OUTPUT);

}

}

/******************************************************************************************************************

// When alwaysOn_ is true, shouldStartVibrating will always return true,

// meaning the belt will continuously start vibrating in the update method as

// the setReports loop iterates.

// Concurrently, shouldStopVibrating will always return false, ensuring the

belt remains in a vibrating state during consistent head readings.

*/

void CompassBelt::setAlwaysOn(bool alwaysOn) {

alwaysOn_ = alwaysOn;

}

/******************************************************************************************************************

// setAlwaysOn sets the alwaysOn_ flag based on the double press (in main.ino program)

isAlwaysOn simply returns the current state of alwaysOn_. (like a question)

*/

bool CompassBelt::isAlwaysOn() {

return alwaysOn_;

}

/******************************************************************************************************************

// Turns off the haptic feedback for the belt.

// This function calls the 'off' method of the HapticBelt object,

ensuring that any ongoing vibrations are stopped immediately.

*/

void CompassBelt::off()

{

_haptic.off();

}

/******************************************************************************************************************

//goes around the LED's or vibration motors one time, switching on every one , so see, if all are working

//sometimes, there a bad solders or connection or motor is not tightly presses to skin , so it doesn't feel

// also with LED's there had been problems with the dupont connectors used.

// this is started with long press of the pushbutton

*/

void CompassBelt::lampTest() {

int numberOfPins = 16;

for (int i = 0; i < numberOfPins; i++) {

for (int j = 0; j < numberOfPins; j++) {

_bank.digitalWrite(j,LOW);//changed to high

}

_bank.digitalWrite(i,HIGH);//change to low

delay(200);

}

_bank.digitalWrite(numberOfPins - 1,LOW);//change to high

}

/*

// Direction Change & Initiation: If the heading value results in a new direction

//  (calculated and snapped to nearest 18 degrees), the compass belt will update

//  the new direction and turn on the belt. This resets lastOn_ to the current time

//  returned by millis().

*/

void CompassBelt::update(double heading, int degree_shift)

{

if (heading < 0 || heading >= 360) {

return;

}

const double direction_threshold = 3;

double direction = round(heading / 22.5) * 22.5;

//Serial.print("direction: ");

//  Serial.println(direction);

if (lastDirection_ != -1 && abs(direction - heading) > direction_threshold)

/* The choice of -1 for sentinel variable lastDirection_ is often due to several reasons:

// Sentinel Value: -1 is used as a sentinel value to represent an invalid or uninitialized

//  state since direction  values are usually non-negative. It helps distinguish between a real

//  direction (e.g., 0, 1, 2, ...) and a state where no direction has been set.

//

// Ease of Checking: Checking if a variable is equal to -1 is straightforward and unambiguous,

//  which makes the code easier to read and maintain.

//

// Common Convention: Using -1 or other negative values as sentinel values is a common

//  programming practice, making the code more understandable to other developers who encounter it.

*/

{

direction = lastDirection_;

//Serial.print(" Heading change too large, keeping last direction: ");

}

if (direction != lastDirection_) {

if (lastDirection_ != -1) {

_haptic.off(lastDirection_);

}

//Serial.println(direction);

lastDirection_ = direction;

_haptic.on(direction);

lastOn_ = millis();

}

if (shouldStartVibrating()) {

_haptic.on(direction);

lastOn_ = millis();

} else if (shouldStopVibrating()) {

_haptic.off(direction);

}

}

//******************************************************************************************************************

// Vibration should stop if the alwaysOn_ flag is false

//  and enough time has passed since lastOn_ to exceed the onDuration_.

bool CompassBelt::shouldStopVibrating() {

return !alwaysOn_ && millis() - lastOn_ >= onDuration_;

}

bool CompassBelt::shouldStartVibrating() {

unsigned long timeSinceOn = millis() - lastOn_;

return alwaysOn_ || timeSinceOn >= onDuration_ + vibrationInterval_;

}

/******************************************************************************************************************

// Vibration should stop if the alwaysOn_ flag is false

//  and enough time has passed since lastOn_ to exceed the onDuration_.

*/

// Method to set the next vibration interval cyclically

void CompassBelt::setNextVibrationInterval() {

static int currentIndex = 0;

currentIndex = (currentIndex + 1) % (sizeof(vibrationIntervalChoices) / sizeof(vibrationIntervalChoices[0]));

vibrationInterval_ = vibrationIntervalChoices[currentIndex];

}

// Content from CompassBelt.h

#ifndef COMPASSBELT_H

#define COMPASSBELT_H

#include <Arduino.h>

#include <SPI.h>

#include <MCP23S17.h>

#include "HapticBelt.h"

class CompassBelt {

public:

CompassBelt(unsigned long vibrationDuration,

unsigned long vibrationInterval,

uint8_t chipSelect = 0);

void update(double heading, int degree_shift);

void begin();

//    void update_old(float heading, int degree_shift = 0);

void setNextVibrationInterval();

void lampTest();

void off();

bool isAlwaysOn();

void setAlwaysOn(bool alwaysOn);

void setupPins();

private:

HapticBelt* belt_;

unsigned long _vibrationDuration;

unsigned long _vibrationInterval;

unsigned long onDuration_;

unsigned long lastOn_;

uint8_t _chipSelect;

MCP23S17 _bank;

HapticBelt _haptic;

static const long vibrationIntervalChoices[];

unsigned long vibrationInterval_;

bool alwaysOn_ = false;

bool shouldStopVibrating();

bool shouldStartVibrating();

double lastDirection_;

};

#endif

// Content from HapticBelt.cpp

#include "HapticBelt.h"

HapticBelt::HapticBelt(MCP23S17& bank)

: _bank(bank)

{

}

void HapticBelt::begin() {

for (int i = 0; i < 16; i++){

_bank.pinMode(i, OUTPUT);

_bank.digitalWrite(i, LOW);//changed to

}

//Serial.println(" HapticBelt::begin completed");

}

/* Method to check if the direction is valid (used inside  HapticBelt and off)

// VALIDATION

//The direction must be greater than or equal to 0.

//The direction must be less than 360.

//The direction must be a multiple of 22.5.

// Valid directions:

*/

bool HapticBelt::isDirectionValid(double direction)

{

double check_direction = direction *2;

return check_direction >= 0 && check_direction < 720 && (int)check_direction % 45 == 0;

}

/*  Method used in CompassBelt like this: belt_->on(direction, 255);

CHOSEN DIRECTION "ON"*/

void HapticBelt::on(double direction)

{

if (!isDirectionValid(direction)) {

return;

}

double position = (direction / 22.5);

//  Serial.print("position: ");

//  Serial.println(position);

uint8_t pos_int = static_cast<uint8_t>(position);

//It seems that position is from 0 to 15

_bank.digitalWrite(pos_int, HIGH);//changed

Serial.println(pos_int); //xxxx

}

/* Used in CompassBelt like this: belt_->off(direction);

chosen direction "OFF" */

void HapticBelt::off(double direction)

{

if (!isDirectionValid(direction)) {

return;

}

double position = (int)(direction / 22.5);

uint8_t pos_int = static_cast<uint8_t>(position);

_bank.digitalWrite(pos_int, LOW);//change

}

void HapticBelt::off()

{

for (int i = 0; i < 16; i++) {

_bank.digitalWrite(i, LOW);//change low

//  Serial.println(" HapticBelt::off - All Motors OFF");

}

}

// Content from HapticBelt.h

#ifndef HAPTICBELT_H

#define HAPTICBELT_H

#include <Arduino.h>

#include <SPI.h>

#include <MCP23S17.h>

#include <Math.h>

class HapticBelt

{

public:

// Constructor takes an already-existing MCP23S17

HapticBelt(MCP23S17& bank);

void begin();

//   void on_old();

//   void off_old();

void on(double direction);

void off(double direction);

void off();

private:

MCP23S17& _bank;

bool isDirectionValid(double direction);

};

#endif

// Content from main.cpp

#include <Arduino.h>

#include "Compass.h"

#include "Compassbelt.h"

#include "HapticBelt.h"

#include "Button.h"

#include <Wire.h>

#include <SparkFun_BNO08x_Arduino_Library.h>

#include "potentiometer.h"

#define BNO08X_RESET -1

#define TIMEOUT_BOOT_BNO 100

#define BNO08X_I2C_ADDRESS 0x4B

unsigned long millisOld;

const unsigned long vibrationDurationMillis = 200UL;

const unsigned long vibrationIntervalMillis = 1000UL;

const int PIN_SDA = A4;   // works on many cores; otherwise set explicit pin numbers

const int PIN_SCL = A5;

static uint32_t lastGoodEventMs = 0;

static uint32_t bootMs = 0;

static uint32_t lastRecoverAttemptMs = 0;

// Tune these:

const uint32_t STARTUP_GRACE_MS = 3000;          // wait after setup before any recovery

const uint32_t NO_EVENT_BEFORE_RECOVER_MS = 4000; // must be "dead" this long before recovery

const uint32_t RECOVER_COOLDOWN_MS = 5000;        // minimum time between recovery attempts

int degree_shift;

long lastSerialRecv = -10000L;

float lastSerialHeading = 0.0f;

const int alwaysOnButtonPin = 6;

Compass compass;

BNO08x imu;

static inline void wdtSync() {

while (WDT->STATUS.bit.SYNCBUSY) { }

}

void initWatchdog_2s() {

// Route 1kHz clock to WDT (required on SAMD21)

GCLK->GENDIV.reg  = GCLK_GENDIV_ID(4) | GCLK_GENDIV_DIV(32); // 32kHz/32 = 1kHz

GCLK->GENCTRL.reg = GCLK_GENCTRL_ID(4) |

GCLK_GENCTRL_SRC_OSCULP32K |

GCLK_GENCTRL_GENEN;

while (GCLK->STATUS.bit.SYNCBUSY) { }

GCLK->CLKCTRL.reg = GCLK_CLKCTRL_ID_WDT |

GCLK_CLKCTRL_GEN_GCLK4 |

GCLK_CLKCTRL_CLKEN;

while (GCLK->STATUS.bit.SYNCBUSY) { }

// Disable WDT before config

WDT->CTRL.reg = 0;

wdtSync();

// Set ~2s period (2048 cycles @ 1kHz)

//  WDT->CONFIG.reg = WDT_CONFIG_PER(WDT_CONFIG_PER_2048);

//  WDT->CONFIG.reg = WDT_CONFIG_PER(WDT_CONFIG_PER_2048_Val);

WDT->CONFIG.reg = WDT_CONFIG_PER(0xB); // 2048 cycles is typically enum value 0xB on SAMD2  wdtSync();

// Optional: early warning interrupt disabled

WDT->EWCTRL.reg = 0;

wdtSync();

// Enable WDT

WDT->CTRL.reg = WDT_CTRL_ENABLE;

wdtSync();

}

static inline void feedWatchdog() {

WDT->CLEAR.reg = WDT_CLEAR_CLEAR_KEY;

wdtSync();

}

void initBNO085() {

delay(1000);

// Re-start the IMU and re-enable the report

if (!imu.begin()) {

Serial.println("BNO08x re-init failed");

return;

}

Compass::setReports(&imu, SH2_GAME_ROTATION_VECTOR, compass.getReportInterval());

Serial.println("BNO08x re-init OK");

}

bool i2cBusClear() {

pinMode(PIN_SDA, INPUT_PULLUP);

pinMode(PIN_SCL, INPUT_PULLUP);

delayMicroseconds(5);

// If SCL is stuck low, we can't clear it here

if (digitalRead(PIN_SCL) == LOW) return false;

// If SDA is stuck low, try to free it by clocking SCL

if (digitalRead(PIN_SDA) == LOW) {

pinMode(PIN_SCL, OUTPUT);

for (int i = 0; i < 9; i++) {

digitalWrite(PIN_SCL, HIGH);

delayMicroseconds(5);

digitalWrite(PIN_SCL, LOW);

delayMicroseconds(5);

}

pinMode(PIN_SCL, INPUT_PULLUP);

delayMicroseconds(5);

}

// Generate a STOP: SDA low -> SCL high -> SDA high

pinMode(PIN_SDA, OUTPUT);

digitalWrite(PIN_SDA, LOW);

delayMicroseconds(5);

pinMode(PIN_SCL, INPUT_PULLUP);

delayMicroseconds(5);

pinMode(PIN_SDA, INPUT_PULLUP);

delayMicroseconds(5);

return (digitalRead(PIN_SDA) == HIGH);

}

void reinitWire() {

Wire.end();

delay(10);

Wire.begin();

Wire.setClock(10000);

Wire.setTimeout(50);

}

CompassBelt  compassBelt(vibrationDurationMillis, vibrationIntervalMillis,7);   // chip select = 0

void sensorValueToDegree(int &degree_shift);

Button buttonInstance(alwaysOnButtonPin, &compassBelt);

void waitForUser() {

Serial.begin(9600);

while (!Serial) {

; // wait for Serial Monitor to open

}

Serial.println("=== Waiting for user ===");

Serial.println("Send any character to start setup");

while (!Serial.available()) {

; // wait for user input

}

Serial.read(); // consume char

Serial.println("=== Starting setup ===");

}

void setup() {

//waitForUser();

delay(5000);  // give bootloader time to enumerate

Serial.begin(9600);

unsigned long t0 = millis();

bool serialReady = false;

while (millis() - t0 < 2000) {        // Wait max 2 sec

if (Serial) {

serialReady = true;

break;

}

delay(10);

}

if (serialReady) {

Serial.println("Serial monitor connected!");

digitalWrite(LED_BUILTIN, HIGH);    // optional indicator

} else {

// Serial monitor not connected, continue anyway

}

Wire.begin();

Wire.setClock(10000);

delay(200);   // IMPORTANT: let BNO085 boot

if (!imu.begin())

{

Serial.println("BNO08x not detected!");

while (1);

Serial.println("BNO08x ready");

}

initWatchdog_2s();

if (!imu.enableGameRotationVector(50)) {

Serial.println("❌ enableGameRotationVector failed");

} else {

Serial.println("✅ GameRotationVector enabled");

}

buttonInstance.setup();

compassBelt.setupPins();

Serial.println("Setup complete");

bootMs = millis();

lastGoodEventMs = millis();        // start "alive"

lastRecoverAttemptMs = 0;

}

void loop() {

//Serial.println(digitalRead(alwaysOnButtonPin));

buttonInstance.tick();

delay(10);

bool gotEvent = imu.getSensorEvent();

if (gotEvent) {

Serial.print("✅ Event sensorId=");

Serial.println(imu.sensorValue.sensorId);

compass.updateFromSensor(imu.sensorValue);  // ← pass event

lastGoodEventMs = millis();

} else {

static uint32_t lastPrintMs = 0;

if (millis() - lastPrintMs > 1000) {

Serial.println("No sensor event (not necessarily an error)");

lastPrintMs = millis();

}

}

if (gotEvent) {

float heading = compass.getHeading(nullptr, degree_shift);

uint8_t acc = compass.getAccuracy();

Serial.print("Heading: ");

Serial.print(heading);

Serial.print("  Accuracy: ");

Serial.println(acc);

}

// ✅ Do NOT attempt recovery during startup grace period

if (millis() - bootMs < STARTUP_GRACE_MS) {

feedWatchdog();

return;

}

// ✅ Only recover if we've had no events for long enough

bool deadTooLong = (millis() - lastGoodEventMs) > NO_EVENT_BEFORE_RECOVER_MS;

// ✅ And don't try recovery too frequently

bool cooldownOver = (lastRecoverAttemptMs == 0) ||

((millis() - lastRecoverAttemptMs) > RECOVER_COOLDOWN_MS);

if (deadTooLong && cooldownOver) {

lastRecoverAttemptMs = millis();

Serial.println("⚠️ No events for too long -> attempting recovery");

i2cBusClear();

reinitWire();

delay(1000);     // <-- your requested "add some seconds"

initBNO085();    // calls imu.begin() + setReports

}

feedWatchdog();

sensorValueToDegree(degree_shift);

float heading = compass.getHeading(&imu, degree_shift);

uint8_t acc = compass.getAccuracy();

Serial.print("Heading: ");

Serial.print(heading);

Serial.print("  Accuracy: ");

Serial.println(acc);

compassBelt.update(heading, degree_shift);

}

// Content from potentiometer.cpp

#include "potentiometer.h"

#include <Arduino.h>

void sensorValueToDegree(int &degree_shift) {

int sensorValue = analogRead(A3);

degree_shift = int(sensorValue / 2.86666);

if (degree_shift > 360) {

degree_shift = 360;

}

}

// Content from potentiometer.h

#ifndef POTENTIOMETER_H

#define POTENTIOMETER_H

void sensorValueToDegree(int &degree_shift);

#endif