// Content from Compass.cpp

#include "Compass.h"

void Compass::quaternionToEuler(float qr, float qi, float qj, float qk, euler_t* ypr, bool degrees) {

float sqr = sq(qr);

float sqi = sq(qi);

float sqj = sq(qj);

float sqk = sq(qk);

ypr->yaw = atan2(2.0 * (qi * qj + qk * qr), (sqi - sqj - sqk + sqr));

ypr->pitch = asin(-2.0 * (qi * qk - qj * qr) / (sqi + sqj + sqk + sqr));

ypr->roll = atan2(2.0 * (qj * qk + qi * qr), (-sqi - sqj + sqk + sqr));

if (degrees) {

ypr->yaw *= RAD_TO_DEG;

ypr->pitch *= RAD_TO_DEG;

ypr->roll *= RAD_TO_DEG;

}

}

float Compass::getNorthDirection(float yaw) {

if (yaw < 0) {

return (360 + yaw);

} else {

return yaw;

}

}

float Compass::ConvertToShownDirection(float AbsDir, int degree_shift) {

return ((int(AbsDir) + degree_shift) % 360);

}

void Compass::quaternionToEulerRV(sh2_RotationVectorWAcc_t* rotational_vector,

euler_t* ypr,

bool degrees) {

quaternionToEuler(rotational_vector->real, rotational_vector->i, rotational_vector->j, rotational_vector->k, ypr, degrees);

}

void Compass::quaternionToEulerGI(sh2_GyroIntegratedRV_t* rotational_vector,

euler_t* ypr,

bool degrees) {

quaternionToEuler(rotational_vector->real, rotational_vector->i, rotational_vector->j, rotational_vector->k, ypr, degrees);

}

void Compass::updateFromSensor(const sh2_SensorValue_t& value) {

sensorValue_1 = value;

update_sensor_1 = true;

}

float Compass::getHeading(BNO08x* imu, int degree_shift) {

if (update_sensor_1) {

if (reportType == SH2_ROTATION_VECTOR) {

quaternionToEulerRV(&sensorValue_1.un.rotationVector, &ypr_1, true);

} else if (reportType == SH2_GYRO_INTEGRATED_RV) {

quaternionToEulerGI(&sensorValue_1.un.gyroIntegratedRV, &ypr_1, true);

}

update_sensor_1 = false;

}

float absDir = getNorthDirection(ypr_1.yaw);

float shownDir = ConvertToShownDirection(absDir, degree_shift);

return shownDir;

}

void Compass::setReports(BNO08x* imu, sh2_SensorId_t reportType, long report_interval)

{

if (!imu->enableReport(reportType, report_interval))

{

Serial.println("Could not enable rotation vector on BNO08x");

}

}

// Content from Compass.h

#ifndef COMPASS_H

#define COMPASS_H

#include <Arduino.h>

#include <Wire.h>

#include <sh2_SensorValue.h>

#include <SparkFun_BNO08x_Arduino_Library.h>

struct euler_t {

float yaw;

float pitch;

float roll;

};

class Compass {

public:

Compass() : update_sensor_1(false),

reportType(SH2_ROTATION_VECTOR),

reportIntervalUs(100000L) {}

float getHeading(BNO08x* bno08x, int degree_shift);

void updateFromSensor(const sh2_SensorValue_t& value);

uint8_t getAccuracy() const { return sensorValue_1.status; }  // <-- ADD THIS

long getReportInterval() const { return reportIntervalUs; }

static void setReports(BNO08x* bno08x, sh2_SensorId_t reportType, long report_interval);

private:

euler_t ypr_1;

sh2_SensorValue_t sensorValue_1;

bool update_sensor_1;

sh2_SensorId_t reportType;

long reportIntervalUs;

static void quaternionToEuler(float qr, float qi, float qj, float qk, euler_t* ypr, bool degrees);

static void quaternionToEulerRV(sh2_RotationVectorWAcc_t* rotational_vector, euler_t* ypr, bool degrees);

static void quaternionToEulerGI(sh2_GyroIntegratedRV_t* rotational_vector, euler_t* ypr, bool degrees);

float getNorthDirection(float yaw);

float ConvertToShownDirection(float AbsDir, int degree_shift);

};

#endif // COMPASS_H

// Content from main.cpp

#include <Arduino.h>

#include "Compass.h"

#include <Wire.h>

#include <SparkFun_BNO08x_Arduino_Library.h>

#define BNO08X_RESET -1

#define TIMEOUT_BOOT_BNO 100

#define BNO08X_I2C_ADDRESS 0x4B

unsigned long millisOld;

const unsigned long vibrationDurationMillis = 200UL;

const unsigned long vibrationIntervalMillis = 1000UL;

const unsigned long serialKeepDurationMillis = 10UL * 1000UL;

const int PIN_SDA = A4;   // works on many cores; otherwise set explicit pin numbers

const int PIN_SCL = A5;

static uint32_t lastGoodEventMs = 0;

static uint32_t bootMs = 0;

static uint32_t lastRecoverAttemptMs = 0;

// Tune these:

const uint32_t STARTUP_GRACE_MS = 3000;          // wait after setup before any recovery

const uint32_t NO_EVENT_BEFORE_RECOVER_MS = 4000; // must be "dead" this long before recovery

const uint32_t RECOVER_COOLDOWN_MS = 5000;        // minimum time between recovery attempts

int degree_shift;

long lastSerialRecv = -10000L;

float lastSerialHeading = 0.0f;

Compass compass;

BNO08x imu;

static inline void wdtSync() {

while (WDT->STATUS.bit.SYNCBUSY) { }

}

void initWatchdog_2s() {

// Route 1kHz clock to WDT (required on SAMD21)

GCLK->GENDIV.reg  = GCLK_GENDIV_ID(4) | GCLK_GENDIV_DIV(32); // 32kHz/32 = 1kHz

GCLK->GENCTRL.reg = GCLK_GENCTRL_ID(4) |

GCLK_GENCTRL_SRC_OSCULP32K |

GCLK_GENCTRL_GENEN;

while (GCLK->STATUS.bit.SYNCBUSY) { }

GCLK->CLKCTRL.reg = GCLK_CLKCTRL_ID_WDT |

GCLK_CLKCTRL_GEN_GCLK4 |

GCLK_CLKCTRL_CLKEN;

while (GCLK->STATUS.bit.SYNCBUSY) { }

// Disable WDT before config

WDT->CTRL.reg = 0;

wdtSync();

// Set ~2s period (2048 cycles @ 1kHz)

//  WDT->CONFIG.reg = WDT_CONFIG_PER(WDT_CONFIG_PER_2048);

//  WDT->CONFIG.reg = WDT_CONFIG_PER(WDT_CONFIG_PER_2048_Val);

WDT->CONFIG.reg = WDT_CONFIG_PER(0xB); // 2048 cycles is typically enum value 0xB on SAMD2  wdtSync();

// Optional: early warning interrupt disabled

WDT->EWCTRL.reg = 0;

wdtSync();

// Enable WDT

WDT->CTRL.reg = WDT_CTRL_ENABLE;

wdtSync();

}

static inline void feedWatchdog() {

WDT->CLEAR.reg = WDT_CLEAR_CLEAR_KEY;

wdtSync();

}

bool readBNO085() {

// For SparkFun BNO08x library, a "read" is typically: imu.getSensorEvent()

// Return true if we got a new event, false otherwise.

return imu.getSensorEvent();

}

void initBNO085() {

delay(1000);

// Re-start the IMU and re-enable the report

if (!imu.begin()) {

Serial.println("BNO08x re-init failed");

return;

}

Compass::setReports(&imu, SH2_ROTATION_VECTOR, compass.getReportInterval());

Serial.println("BNO08x re-init OK");

}

bool i2cBusClear() {

pinMode(PIN_SDA, INPUT_PULLUP);

pinMode(PIN_SCL, INPUT_PULLUP);

delayMicroseconds(5);

// If SCL is stuck low, we can't clear it here

if (digitalRead(PIN_SCL) == LOW) return false;

// If SDA is stuck low, try to free it by clocking SCL

if (digitalRead(PIN_SDA) == LOW) {

pinMode(PIN_SCL, OUTPUT);

for (int i = 0; i < 9; i++) {

digitalWrite(PIN_SCL, HIGH);

delayMicroseconds(5);

digitalWrite(PIN_SCL, LOW);

delayMicroseconds(5);

}

pinMode(PIN_SCL, INPUT_PULLUP);

delayMicroseconds(5);

}

// Generate a STOP: SDA low -> SCL high -> SDA high

pinMode(PIN_SDA, OUTPUT);

digitalWrite(PIN_SDA, LOW);

delayMicroseconds(5);

pinMode(PIN_SCL, INPUT_PULLUP);

delayMicroseconds(5);

pinMode(PIN_SDA, INPUT_PULLUP);

delayMicroseconds(5);

return (digitalRead(PIN_SDA) == HIGH);

}

void reinitWire() {

Wire.end();

delay(10);

Wire.begin();

Wire.setClock(10000);

Wire.setTimeout(50);

}

void sensorValueToDegree(int &degree_shift);

void setup() {

delay(5000);

Serial.begin(9600);

unsigned long t0 = millis();

while (millis() - t0 < 2000) {

if (Serial) break;

delay(10);

}

if (Serial) Serial.println("Serial monitor connected!");

Wire.begin();

Wire.setClock(10000);

Wire.setTimeout(50);

delay(200); // let BNO08x boot

if (!imu.begin()) {

Serial.println("BNO08x not detected!");

while (1) { }

}

initWatchdog_2s();

//Compass::setReports(&imu, SH2_ROTATION_VECTOR, compass.getReportInterval());

if (!imu.enableGameRotationVector(50)) {

Serial.println("❌ enableGameRotationVector failed");

} else {

Serial.println("✅ GameRotationVector enabled");

}

Serial.println("Setup complete");

bootMs = millis();

lastGoodEventMs = millis();        // start "alive"

lastRecoverAttemptMs = 0;

}

void loop() {

bool gotEvent = imu.getSensorEvent();

if (gotEvent) {

Serial.print("✅ Event sensorId=");

Serial.println(imu.sensorValue.sensorId);

compass.updateFromSensor(imu.sensorValue);  // ← pass event

lastGoodEventMs = millis();

} else {

static uint32_t lastPrintMs = 0;

if (millis() - lastPrintMs > 1000) {

Serial.println("No sensor event (not necessarily an error)");

lastPrintMs = millis();

}

}

if (gotEvent) {

float heading = compass.getHeading(nullptr, degree_shift);

uint8_t acc = compass.getAccuracy();

Serial.print("Heading: ");

Serial.print(heading);

Serial.print("  Accuracy: ");

Serial.println(acc);

}

// ✅ Do NOT attempt recovery during startup grace period

if (millis() - bootMs < STARTUP_GRACE_MS) {

feedWatchdog();

return;

}

// ✅ Only recover if we've had no events for long enough

bool deadTooLong = (millis() - lastGoodEventMs) > NO_EVENT_BEFORE_RECOVER_MS;

// ✅ And don't try recovery too frequently

bool cooldownOver = (lastRecoverAttemptMs == 0) ||

((millis() - lastRecoverAttemptMs) > RECOVER_COOLDOWN_MS);

if (deadTooLong && cooldownOver) {

lastRecoverAttemptMs = millis();

Serial.println("⚠️ No events for too long -> attempting recovery");

i2cBusClear();

reinitWire();

delay(1000);     // <-- your requested "add some seconds"

initBNO085();    // calls imu.begin() + setReports

}

feedWatchdog();

float heading = compass.getHeading(&imu, degree_shift);

uint8_t acc = compass.getAccuracy();

Serial.print("Heading: ");

Serial.print(heading);

Serial.print("  Accuracy: ");

Serial.println(acc);

}